APPLICATION: ROOFING

##### ASSEMBLY: FM APPROVED ASSEMBLY

##### GARDEN ROOF

##### W/FABRIC REINFORCED

MM 6125 (215 MILS)

FM APPROVED GARDEN ROOF ASSEMBLY

LONG FORM SPECIFICATION

Note to Specifier: This Guide Specification is suitable for continental US projects; for projects located in Alaska, Hawaii, and Puerto Rico, please contact Hydrotech for assistance.

PART 1 GENERAL

- SUMMARY

  - This specification serves as a guideline and shall be adapted to suit the needs of each individual project by the architect. It is prepared in accordance with the CSI Master Specification format and shall be included as a separate section under DIVISION 07 Thermal and Moisture Protection. Improvements and other changes to the contents shall be made only with the written approval of the architect.

- RELATED SECTIONS (Note to Specifier: Edit to project requirements)

- DIVISION 03 - Concrete [Section 03 00 00] - Roof Deck Surface/Substrate. The coordination of this section is necessary to facilitate the successful installation of the waterproofing membrane.

    - Cast in Place Concrete/Composite Deck

      - Strength/density: 	minimum 2,500 psi (17,235 kPa) compressive strength

minimum 115 pcf (1842 kg/m3) density

      - Finish:			Broom finish or equivalent similar to ICRI CSP 3 to 5.  Steel			troweled finish is not desirable.

    - Concrete Hydration (Cure):

      - Method of Cure:  Water cure, wet coverings, paper sheets, plastic sheets, or approved liquid curing compound (sodium silicate preferred).

      - Duration of Cure/Dry:

        - Structural Weight Concrete: recommend 28 days, minimum 14 days, prior to application of the membrane.

        - Lightweight Structural Concrete:  recommend 60 days, minimum 28 days, prior to application of membrane. Venting of the deck from the underside is recommended to facilitate drying.

        - The above minimum cure/dry times are recommended based upon basic concrete fundamentals and experience. Depending on conditions (i.e., ambient temperature, humidity) the concrete may be dry enough to receive application of the membrane in less than the 14-day minimum recommendation. Consult Hydrotech for specifics when less than the minimum is desired.

    - Refer to SECTION 3.02 PREPARATION for additional deck types.

  - Division 07– Membrane Roofing- [Section 07 50 00] Protected Membrane Roofing as supplied by American Hydrotech, Inc.

  - Division 32 Exterior Improvements [Section 32 14 00 / 32 33 00] - Paving/Site Furnishings as supplied by American Hydrotech, Inc.

  - Other Related Sections:

    - Section 05 30 00 – Metals- Metal roof deck surface/ substrate.

    - Section 06 00 00 – Woods, Plastic, and Composites- Wood blocking, curbing, and roof deck/ substrate.

    - Section 07 03 00 – Flashing and Sheet Metal- Sheet metal flashing and counterflashing

    - Section 07 72 00 – Roof Accessories- Curbs, vents, and hatches.

    - Section 07 92 00 – Joint Sealants- Caulking and sealants.

    - Section 22 00 00 – Plumbing- Water source

    - Section 31 22 00 – Grading- Fine grading of media.

    - Section 32 80 00 – Irrigation

    - Section 32 90 00 – Planting- Turf, trees, shrubs, groundcovers, bulbs.

    - Section 32 94 00 – Planting Accessories- Metal edging by Green Roof Solutions.

    - Section 33 00 00 – Utilities

- REFERENCES

  - American Society for Testing and Materials (ASTM).

  - Canadian General Standards Board, CGSB-37.50-M89, Standard for Asphalt, Rubberized, Hot Applied, for Roofing and Waterproofing.

  - Underwriters Laboratories (UL) Class A.

  - ANSI/SPRI VR-1 2011 “Procedure for Investigating Resistance to Root Penetration on Vegetative Roofs”.

  - International Concrete Repair Institute (ICRI) Concrete Surface Profile (CSP) Scale.

- DEFINITIONS

  - Green Roof – An area of planting/landscaping, built up on a waterproofed substrate at any level that is separated from the natural ground by a man-made structure.

  - Extensive Green Roof – Low maintenance landscaping consisting of shallow growing media depths (< 6 inches (152mm)) with plant varieties restricted to primarily mosses, herbs, and succulents capable of withstanding harsh growing conditions.

  - Intensive Green Roof – Landscaping requiring regular maintenance, consisting of deeper growing media depths (> 6 inches (152mm)) with a wider variety of plant species possible including shrubs and small trees.

  - Lawn Green Roof – Lawn oriented landscaping requiring at-grade maintenance. Can include sodded or seeded turfgrasses or naturalized grasses with growing media depths > 8 inches (203mm).

  - Garden Roof® – Patented system of drainage, water retention and root barrier components utilized in the construction of green roofs over Hydrotech’s MM 6125® roofing membrane.

  - Steep Slope Green Roof – Defined as a slope exceeding 3:12 pitch.

  - “C” Factor – The runoff coefficient used in the Rational Method; “C” represents the portion of the storm rainfall that becomes runoff.

  - Curve Number (CN) – A number that is used with Natural Resource Conservation Service (SCS) methods to convert rainfall depth into runoff volume. The Curve Number considers a site’s soil type, plant cover, impervious cover, interception, and surface storage.

- SYSTEM DESCRIPTION

  - Furnish and install a completed Extensive Garden Roof® Assembly including Monolithic Membrane 6125®-FR and flashings, separation/ root barrier protection, prefabricated drainage course, Styrofoam™ brand insulation, drainage/water retention component, rock mineral wool (if required), filter fabric, lightweight engineered growing media and vegetation.

- SUBMITTALS

  - Certification from an approved independent testing laboratory experienced in testing rubberized asphalt material, that the material meets the CGSB-37.50-M89 standard for rubberized asphalt membranes, including applicable ASTM procedures.

  - Certification that the roofing membrane has current validation by Underwriters Laboratories, or other approved independent validation service provider, of a minimum 40% recycled content (post-consumer).

  - Certification that the roof membrane assembly is currently Class A listed with Underwriters Laboratories.

  - Certification showing full time quality control of production facilities responsible for the manufacture of the rubberized asphalt and that each batch of material is tested to ensure conformance with the manufacturers published physical properties.

  - Certification that the plant manufacturing the rubberized asphalt material has IS0 9001-2015 approval as evidenced by a copy of the official certificate.

  - Certification showing that all components of the green roof assembly are being supplied and warranted by a single-source manufacturer.

  - Certification that the extruded polystyrene insulation if used is free from CFC's.

  - Ballasting requirements for the specified loose laid extruded polystyrene insulation, and as referenced in part 1.09.K., shall be provided to include the following:

    - A written ballast review on membrane manufacturer’s letterhead outlining specific roof level ballasting requirements required to satisfy limited wind resistance warranty conditions.

    - Each roof level shall be individually evaluated and prepared during the design and prebid process.

    - A final ballast review shall be submitted that reflects the designed conditions at the time of the project bid.

  - Provide product data on all components of the green roof assembly.

  - Stormwater performance of the specific green roof assembly for the project shall be provided and include: (Note to Specifier: insert ## specific to given project)

    - Composite Curve Number (CN) shall be ##.

    - Composite C factor shall be ##.

    - Total volume of water stored in the growing media shall be a minimum of ## cubic feet.

    - Total volume of water stored in the water retention/drainage element shall be ## cubic feet.

    - Hydrograph of vegetated roof system showing stormwater release delay and stormwater volume reduction.

    - Data for total water retention capacity of the engineered growing media when tested to ASTM E2399.

    - Data for total water retention capacity of the rock mineral wool components when tested to ASTM E2397.

  - Evidence indicating that water is available at the roof level to ensure that the vegetation can receive sufficient moisture through proper maintenance of the green roof.

- QUALITY ASSURANCE

  - The Roofing/Waterproofing Contractor shall demonstrate qualifications to perform the work of this Section by submitting the following documentation:

    - Certification or license by the membrane manufacturer as a locally based, authorized applicator of the product the installer intends to use, for a minimum of five (5) years.

    - List of at least three (3) projects, satisfactorily completed within the past five (5) years of similar scope and complexity to this project.  Previous experience submittal shall correspond to specific membrane system proposed for use by applicator.

  - The Green Roof Installing Contractor shall demonstrate qualifications to perform the work of this Section by submitting the following documentation:

    - Certification or license by the green roof assembly supplier as a locally based, authorized applicator of the products the installer intends to use, for a minimum of five (5) years.

    - List of at least three (3) projects, satisfactorily completed within the past five (5) years of similar scope and complexity to this project.  Previous experience submittal shall correspond to specific membrane system proposed for use by applicator.

  - The Green Roof Maintenance Contractor shall demonstrate qualifications to perform the work of this Section by submitting the following documentation:

    - List of at least three (3) green roof projects, satisfactorily completed within the past five (5) years of similar scope and complexity to this project. Previous experience submittal shall correspond to specific membrane system proposed for use by applicator.

  - Refer to Section 1.05 SYSTEM DESCRIPTION.  Include single-source for all components from the manufacturer.

  - The rubberized asphalt membrane product shall contain an inert filler and crumb rubber to enable the product to be resistant to acids (fertilizers, building washes and acid rain) and maintain membrane thickness during application respectively.

  - Membrane Manufacturer shall have available an in-house technical staff to assist the contractor, when necessary, in application of the products and final inspection of the assembly.

  - Membrane Manufacturer Qualifications:  Manufacturer shall demonstrate qualifications to supply materials of this section by certifying the following:

    - Membrane Manufacturer shall show evidence that the specified rubberized asphalt has been manufactured by the same source for thirty-five (35) years and successfully installed on a yearly basis for a minimum of thirty-five (35) years on projects of similar scope and complexity.

    - Membrane Manufacturer shall not issue warranties for terms longer than they have been manufacturing their hot fluid rubberized asphalt membrane.

  - Green Roof Supplier shall show evidence that the specified green roof assembly has been developed, marketed, supported, and installed for a minimum of fifteen (15) years on projects of similar complexity.

  - Green Roof Supplier shall provide data and calculations, specific to the products being submitted, that verify that the green roof assembly specified meets the project’s criteria for storm water runoff volume and rate control.

    - Calculations shall be based on actual testing of supplier’s green roof components to be used for the project including but not limited to the regionally specific growing media formulation and water retention/drainage materials.

    - Calculations shall account for vegetated and un-vegetated portions of the roof as well as local climatic conditions including rainfall depth, intensity, duration, and timing.

  - Green Roof Supplier shall provide data demonstrating that the composite C-factor and Curve Number parameters for the specified green roof assembly are less than or equal to those factors used in the engineering design and analysis for the project’s drainage and storm water systems analysis.

  - Pre-Construction Conferences. The manufacturer will meet with the necessary parties at the jobsite to review and discuss project conditions as it relates to the integrity of the roofing assembly.

- DELIVERY, STORAGE AND HANDLING

  - Deliver materials in original unopened containers of packaging clearly labeled with manufacturer’s name, brand name, instruction for use, all identifying numbers, and U.L. labels.

  - Materials shall be stored in a neat, safe manner, not to exceed the allowable structural capacity of the storage area.

  - Store materials in a clean, dry area protected from water and direct sunlight.

  - Store all adhesives at temperatures between 60°F (15.5°C) and 80°F (26.6°C). If exposed to lower temperatures, restore materials to 60°F (15.5°C) minimum temperature before using.

  - Keep roof substrate board dry before, during, and after installation. Outside storage shall be off ground and protected by a breathable waterproof covering. Roof substrate board shall be roofed the same day as installed.

  - Vegetation shall be handled and stored in accordance with the Hydrotech Extensive Garden Roof Plant Installation and Maintenance Guideline.

- PROJECT CONDITIONS

  - Application of the membrane shall not commence nor proceed during inclement weather.  All surfaces to receive the membrane shall be free of water, dew, frost, snow, and ice.

  - Application of membrane shall not commence nor proceed when the ambient temperature is below 0°F (-17.7°C).

  - Preparation and application of membrane shall be conducted in well-ventilated areas.

  - Over its service life, do not expose membrane or accessories to a constant temperature more than 180°F (82°C) (i.e., hot pipes and vents or direct steam venting, etc.).

  - Adhesives contain petroleum distillates and are extremely flammable.  Do not breathe vapors or use near an open fire.  Do not use in confined areas without adequate ventilation. Consult container or packaging labels and Material Safety Data Sheets (MSDS) for specific safety information.

  - Do not allow waste products (petroleum, grease, oil, solvents, vegetable or mineral oil, animal fat, etc.) to come in contact with the roof membrane.  Any exposure to foreign materials or chemical discharges shall be presented to membrane manufacturer for evaluation to determine any impact on the roof membrane assembly performance.

  - Concrete Deck Surface Condition.  IMPORTANT - Refer to 1.02 RELATED SECTIONS.

  - Deck Preparation; Refer to SECTION 3.02 PREPARATION.

  - Deck slopes greater than 3 inches in 12 inches (approx. 15 degrees or 25%) shall be limited to extensive and lawn applications and require special installation considerations.  (Note to specifier: CONTACT Hydrotech for specifics).

  - The project design professional shall design the roofing/waterproofing assembly to properly locate the dew point condition and determine the need and location for a vapor retarder to prevent excessive condensation contamination within the roofing/waterproofing assembly and/or delamination of the roofing/waterproofing membrane from the substrate.

  - The project design professional shall design the assembly to remain in place withstanding the wind speeds and uplift pressures.

  - General Contractor shall assure that adequate protection is to the membrane and plantings to prevent damage from subsequent trade traffic.

- WARRANTY

  - Upon completion of the work, the contractor shall supply the owner with a single source warranty of U.S. origin direct from the manufacturer if applicable for the project.

  - Each warranty varies in scope and terms. (Note to specifier: CONTACT Hydrotech for exact warranty terms and conditions to meet the specific project requirements).

  - Warranties available from the manufacturer: (Note to Specifier: Edit to project requirements)

    - Material Warranty; excludes labor.

      - Duration: 2-, 5-, 10-, 20-year (Edit as required)

    - Watertightness Warranty: includes labor and material to maintain watertight condition and replacement of Hydrotech supplied roof substrate board.

      - Duration: 5-, 10-, 15-, 20-year (Edit as required)

    - Thermal Warranties; includes 90% retention of the original thermal value.

      - Duration 5-, 10-, 15-, 20-year (Edit as required)

      - Note to specifier: A limited wind resistance warranty, may also be available for the insulation. Contact Hydrotech DURING THE DESIGN PHASE for project specific ballasting assistance or guidelines.

    - Total System Warranties; covers components of the green roof assembly, including membrane, flashing, insulation, Garden Roof® components, vegetation, pavers and Checker Block ballast units. Includes removal and reinstallation of the insulation, Garden Roof® components, vegetation, pavers, Checker Block ballast units and growing media (≤24 inches deep) when supplied, installed, and maintained per Hydrotech’s requirements, and replacement of Hydrotech supplied roof substrate board.

      - Duration of Membrane/Flashing and replacement of Hydrotech supplied roof substrate board from date of installation (watertight condition):

        - 5-, 10-, 15-, 20-year (edit as required).

      - Duration of Insulation from date of purchase:

        - 5-, 10-, 15-, 20-year (edit as required)

        - 90% of original thermal value

        - Note to specifier: A limited wind resistance warranty, may also be available for the insulation.  Contact Hydrotech DURING THE DESIGN PHASE for project specific ballasting assistance or guidelines.

      - Material Integrity of Garden Roof® Components from date of purchase:

        - 5-, 10-, 15-, 20-year (Edit as required)

      - Extensive Vegetation: 2-year thrive and coverage from date of installation:

        - Sedum Plugs: minimum 50% coverage end of year 1; 80% coverage end of year 2.

        - Sedum carpet and sedum tile: minimum 75% coverage at delivery and end of year 1; 90% coverage end of year 2.

        - Perennial Plugs: 2-year thrive from date of installation.

      - Duration of Pavers and Checker Block ballast units from date of purchase will not crack, split, or delaminate due to freeze-thaw)

        - 5-, 10-year (Edit as required)

    - CONTACT HYDROTECH FOR EXACT WARRANTY TERMS AND CONDITIONS.

PART 2- PRODUCTS

- GENERAL

  - Refer to Section 1.05 SYSTEM DESCRIPTION. All components shall be obtained as a single source from the membrane / green roof manufacturer to ensure total system compatibility and integrity.

    - Manufacturer:

American Hydrotech, Inc.

401 Michigan Ave, Suite 1550

Chicago, Illinois 60611

1.800.877.6125 or 312.337.4998

Web Site: www.hydrotechusa.com

- MATERIALS

  - Membrane

Membrane shall be a hot, fluid applied, rubberized asphalt membrane meeting the CGSB-37.50-M89 standard and other pertinent physical properties:

      - American Hydrotech, Inc., Monolithic Membrane 6125 (40% post-consumer recycled content)

| PROPERTY | TEST METHOD | REQUIREMENT |
| --- | --- | --- |
| Flash point | CGSB-37.50-M89 ASTM D-92 | > 500°F* (260°C) |
| Penetration | CGSB-37.50-M89 ASTM D-5329 | @ 77°F (25°C) max. 110 @ 122°F (50°C) max. 200 |
| Flow | CGSB-37.50-M89 ASTM D-5329 | @ 140°F (60°C) max. 3.0mm |
| Toughness | CGSB-37.50-M89 | ≥ 5.5 Joules |
| Ratio of Toughness to Peak Load | CGSB-37.50-M89 | ≥ 0.040 |
| Water Vapor Permeability | CGSB-37.50-M89 ASTM E-96, Procedure E | ≤ 1.7 ng/Pa.s.m2 (0.027 perm) |
| Water Absorption | CGSB-37.50-M89 | Gain in weight 0.35 g max. Loss in weight 0.18 g max. |
| Low Temperature Flexibility (-25°C) | CGSB-37.50-M89 | No delamination, adhesion loss, or cracking |
| Low Temperature Crack Bridging Capability | CGSB-37.50-M89 | No cracking, adhesion loss, or splitting |
| Heat Stability | CGSB-37.50-M89 | No change in viscosity, penetration, flow or low temperature flexibility |
| Viscosity | CGSB-37.50-M89 | 2 - 15 seconds |
| Water Resistance (5 days/50°C) | CGSB-37.50-M89 | No delamination, blistering, emulsification, or deterioration |
| Softening Point | ASTM D-36 | 180°F (82°C) |
| Elongation | ASTM D-5329 | 1000% minimum |
| Resiliency | ASTM D-5329 | 40% minimum |
| Bond to Concrete 0°F (-18°C) | ASTM D-5329 | Pass |
| Resistance to Acid | ASTM D-896 Procedure 7.1 (N-8) | Pass - 50% Nitric Acid Pass - 50% Sulfuric Acid |
| Resistance to Hydrostatic Pressure | ASTM D-08.22 Draft 2 | 100 psi (Equals 231 foot of head water) |
| Resistance to Salt Water (20% Sodium carbonate calcium chloride) | ASTM D-896 similar | No delamination, blistering, emulsification, or deterioration |
| Resistance to Fertilizer (Undiluted 15/5/5, nitrogen/phosphorus/potash) | ASTM D-896 similar | No delamination, blistering, emulsification, or deterioration |
| Resistance to Animal Waste | 3-year exposure | No deterioration |
| Solids Content |  | 100% - no solvents |
| Recycled Content | UL Validated | 40% (post-consumer) (30% REACH compliant) |
| Shelf Life |  | 10 years (sealed) |
| Specific Gravity |  | 1.15 + .02 |
| * Or alternatively not less than 25oC above the manufacturer’s maximum recommended application temperature |  |  |

  - Surface Conditioner

Asphaltic surface conditioner for concrete surfaces only.

      - American Hydrotech, Inc., Surface Conditioner

  - Reinforcing

Spunbonded polyester fabric (standard duty) reinforcing sheet.

      - American Hydrotech, Inc., Flex Flash® F

60-mil (1.5 mm) thick, uncured neoprene (heavy duty) reinforcing sheet.

      - American Hydrotech, Inc., Flex Flash® UN

  - Flashing (Note to Specifier: CONSULT Hydrotech for recommended products and uses.)

60-mil (1.5 mm) thick, uncured neoprene sheet.

      - American Hydrotech, Inc., Flex Flash® UN

157-mil (4 mm) thick, torch-grade, modified asphalt, reinforced flashing membrane.

      - American Hydrotech, Inc., Flex-Flash® MB

Two-component, liquid applied resin membrane flashing system

      - American Hydrotech, Inc., HydroSeal Resin

        - poly methyl-methacrylate (PMMA) resin

      - American Hydrotech, Inc., HydroSeal Matrix

        - acrylic resin with integral chopped polymer fiber reinforcement

      - American Hydrotech, Inc., HydroSeal Flashing Accessories

        - resin based primers, additives, reinforcing fleece, surfacing topcoats

One-component, liquid applied membrane flashing system

      - American Hydrotech, Inc., HydroSeal 1K PU – Membrane

        - Aliphatic and alkaline resistant, low VOC polyurethane (PU) resin

      - American Hydrotech, Inc., HydroSeal 1K accessories

        - Epoxy based primer, fiber reinforcement, fleece

  - Adhesives/Sealant

Contact adhesive to bond elastomeric flashing to an approved substrate.

      - American Hydrotech, Inc., Bonding Adhesive

Tape to bond laps of uncured neoprene flashings

      - American Hydrotech, Inc., Seam Tape

  - Separation/Root Barrier Protection Course (Note to Specifier: Edit to project requirements).

A combination of fiberglass reinforced rubberized asphalt protection sheet and polyethylene root barrier.

      - American Hydrotech, Inc., Hydrocap® 90 and Root Stop.

      - American Hydrotech, Inc., Hydrocap® 90 and Root Stop HD.

  - Prefabricated Drainage Course (Note to Specifier: Consult Hydrotech).

    - Composite drainage system consisting of a three-dimensional, crush-proof, drainage core and a filter fabric.

      - American Hydrotech, Inc., Hydrodrain® 300, 302, 400, 420, 700 or 1000 series.

  - Insulation (Note to Specifier: Edit to project requirements)

    - Extruded polystyrene rigid board insulation for use above membrane.

      - Styrofoam™ brand insulation [TYPE] as manufactured by DuPont™, marketed by American Hydrotech, Inc.

        - Insulation shall meet ASTM C-578, Type VI or VII.

        - Minimum compressive strength, ASTM D-1621, 40 or 60 psi (276 or 414 kPa) (variance by type of product)

        - Maximum water absorption by volume per ASTM C-272, 0.3%

        - Water vapor permeance for 1" product per ASTM E-96, 0.8 perm (max.) (63 ng/Pa/s/m2).

        - Insulation shall have an R value of 5.0°F ft2 h/Btu/in. (0.88 K m2/W) of thickness when tested at 75°F (23.9°C) mean temperature in accordance with ASTM C-518

        - Product shall be free of CFC's

        - Product types available: Styrofoam™ brand, RoofMate; Ribbed RoofMate; PlazaMate; and High Load 100. (Note to Specifier: CONSULT Hydrotech for recommended product type.)

    - Extruded polystyrene rigid board insulation for use as flat fill and tapered insulation below roof substrate board.

      - Styrofoam™ brand insulation [TYPE] as manufactured by DuPont™, marketed by American Hydrotech, Inc.

        - Insulation shall meet ASTM C-578, Type IV.

        - Minimum compressive strength, ASTM D-1621, 25 psi (172.4 kPa).

        - Maximum water absorption by volume per ASTM D-2842, 0.7%.

        - Water vapor permeance for 1" product per ASTM E-96, 1.1 perm (max.) (63 ng/Pa/s/m2)

        - Insulation shall have an R value of 5.0°F ft2 h/Btu/in. (0.88 K m2/W) of thickness when tested at 75°F (23.9°C) mean temperature in accordance with ASTM C-518

        - Product shall be free of CFC's.

        - Product types available:  Styrofoam™ brand, DeckMate Plus, DeckMate Plus FA, Tapered DeckMate Plus FA. (Note to Specifier: CONSULT Hydrotech for recommended product type.)

  - Drainage/Water Retention Component (Note to Specifier:  Edit to project requirements)

    - A three-dimensional, molded recycled polypropylene core with drainage channels on the bottom side and water retention reservoirs and aeration holes on the top side. Non-woven geotextile fabrics are bonded to the top and bottom sides.

      - American Hydrotech, Inc., Gardendrain®

        - Gardendrain GR10

        - Gardendrain GR25

  - Rock Mineral Wool - shall be rock mineral wool batting with the following characteristics:

    - Type:  Needled rock mineral wool

    - Density:  8.43 lbs per cubic foot (135 kg/cubic meter)

    - Thickness:  1.02 inches (26mm)

    - Length of roll:  16.4 feet (5 meters)

    - Roll width:  3.28 feet (39.4 inches; 1 meter)

    - Square feet per roll:  53.8 square feet (5 square meters)

    - Test data for water holding capacity is available on request.

  - Filter Fabric

    - Non-woven, polymeric, geotextile fabric.

      - American Hydrotech, Inc., Systemfilter

  - Growing Media

    - Custom growing media mix capable of supporting vigorous growth of the specified vegetation, complying with the following specification.

      - American Hydrotech, Inc., Intensive LiteTop® Growing Media

| PROPERTY | INTENSIVE LITETOP GROWING MEDIA* |
| --- | --- |
| Grain Size Distribution (ASTM F1632 Method B) |  |
| clay fraction (<0.002mm) | < 3% |
| silt fraction (0.075-0.002mm) | < 12% |
| passing #200 sieve (0.075mm) | < 15% |
| passing #60 sieve (0.25mm) | 5 - 25 % |
| passing #18 sieve (1.0mm) | 20 - 50 % |
| passing #10 sieve (2.0mm) | 30 - 60% |
| passing 1/8-inch sieve | 35 - 70 % |
| passing 1/4-inch sieve | 60 - 95% |
| passing 3/8-inch sieve | 95 - 100 % |
| Density (ASTM E2399) |  |
| Initial Media Density | 55 lbs – 75 lbs/cf |
| Maximum Media Density | 76 lbs – 93 lbs/cf |
| Water/Air Management (ASTM E2399) |  |
| saturated water capacity | > 40% |
| saturated air content | > 10% |
| total pore space | > 45% |
| Water Permeability |  |
| Hydraulic Conductivity | > 10 in/hr |
| pH, Lime, and Salt Content |  |
| pH (saturated paste) | 6.0 – 8.0 |
| EC salts content (water extract) | <3.0 mmhos/cm |
| Organics (LOI 550°C) (ASTM F1647 |  |
| Organic Matter content | 6 – 12 % |
| Compost Fraction |  |
| 1) Meet or exceed USEPA Class A standard, 40 CFR 503.13, Tables 1 & 3 (chemical contaminants) and 40 CFR 503.32(a) (pathogens) and/or be permitted in the state of origin to produce Class A material. |  |
| 2) Meet US Compost Council STA/TMECC criteria or equal for Class I or II stable, mature product. |  |
| *Values shall be adjusted due to availability of local materials or special project conditions related to plant selection and/or environmental conditions. |  |

  - Erosion Control Materials (Note to Specifier: Edit to project requirements, consult Hydrotech).

    - Erosion Control Mat

      - Biodegradable Erosion Control Matting: Composed of straw and/or coconut fiber stitched together with biodegradable thread forming top and bottom netting.

      - American Hydrotech, Inc., GardMat® LT

    - Long Term Erosion Control Mat:  Composed of polypropylene netting.

      - American Hydrotech, Inc., GardMat® N

    - Heavy-Duty Anchors

      - Plastic anchor disk with connected plastic stem and friction-fit plastic top disk used to fasten GardMat® Erosion Control Mat or sedum carpet and tile.

      - American Hydrotech, Inc. Disk Anchors

  - Vegetation/Plantings (Note to Specifier: Edit to project requirements, consult Hydrotech.)

    - Sedum Plugs

      - American Hydrotech, Inc. Garden Roof Extensive Plant Plugs®

      - 2-inch, 50-count plug trays

      - Typical plant list consists of 4-7 sedum species.

      - (Note to specifier: list species here or on plans.)

    - Sedum Cuttings

      - American Hydrotech, Inc. Garden Roof Extensive Sedum Cuttings®

      - Typical plant list consists of 4-7 sedum species.

      - (Note to specifier: list species here or on plans.)

      - Note to specifier: select/specify cutting density:

        - 8 lb/100 sqft

        - 10 lb/100 sqft

        - 12 lb/100 sqft

        - 15 lb/100 sqft

        - Other (specify)

    - Vegetative Carpet

      - American Hydrotech, Inc. Garden Roof® Extensive InstaGreen® Carpet

      - Typical plant list consists of minimum 4-7 sedum species (Consult Hydrotech for current list).

    - Vegetative Tile

      - American Hydrotech, Inc. Garden Roof® Extensive InstaGreen® Tile:

      - InstaGreen Tile: Blend xxxxxx (Note to specifier: Contact Hydrotech for current availability of InstaGreen Tile appropriate for project location and blend type.)

  - Stone Filter Fabric

    - Water permeable polymeric fabric.

      - American Hydrotech, Inc., Stone Filter Fabric

  - Hardscape / Roof Ballast (Note to specifier: Edit to project requirements)

    - Pavers

      - Ballast Pavers

        - American Hydrotech, Inc., Ballast Pavers, meeting the following physical properties:

| PROPERTY | TEST METHOD | VALUES |
| --- | --- | --- |
| Compressive Strength | ASTM C140 | >7,000 psi average min. |
| Flexural Strength | ASTM C293 | >600 psi average min. |
| Water Absorption | ASTM C140 | Not greater than 5% |
| Freeze/Thaw | ASTM C67 | <1% loss/dry weight (50 cycles) |
| Center Load | - | Min. 1,750 lbs. |

      - Architectural Finish Pavers

        - American Hydrotech, Inc., Architectural Pavers, meeting the following physical properties:

| PROPERTY | TEST METHOD | VALUES |
| --- | --- | --- |
| Compressive Strength | ASTM C140 | >7,000 psi average min. |
| Flexural Strength | ASTM C293 | >600 psi average min. |
| Water Absorption | ASTM C140 | Not greater than 5% |
| Freeze/Thaw | ASTM C67 | <1% loss/dry weight (50 Cycles) |
| Center Load | - | Min. 1,750 lbs. |

    - Stone Ballast

      - Well screened and washed stone gravel meeting ASTM D-448-80, gradations #2, 4 or 5 as directed by DuPont™ and American Hydrotech, Inc.

    - Concrete Pour Topping

      - (Note to specifier: DuPont™, manufacturers of Styrofoam™ brand insulation, recommends the incorporation of an air layer between the insulation and concrete.  Hydrotech suggests the use of Hydrodrain AL for this purpose.)

  - Metal Edging

    - Aluminum edging perforated to allow water flow as shown on plans and details.

      - American Hydrotech, Inc. GardenEdge® Metal Edge Restraint; size as noted on plans and details.

      - American Hydrotech, Inc. GardenEdge® Aluminum Leveling Strips: available to accommodate sloped/level roof surfaces.

      - (Note to Specifier:  please indicate size and finish of GardenEdge® metal edging and any leveling strips needed on plans and details. Mill finish aluminum is standard. Color coatings and other metals are available including stainless steel and Cor-Ten weathering steel; contact Hydrotech for further details.)

  - Inspection Chambers

    - Aluminum and stainless steel over drain chambers perforated to allow water flow as shown on plans and details.

      - American Hydrotech, Inc. GardenHatch® Inspection Chambers; size as noted on plans and details.

      - (Note to Specifier: indicate size of GardenHatch® inspection chambers on plans. Custom sizes are available; contact Hydrotech for further details.)

  - Prefabricated Concrete Curbing

High density, hydraulically pressed, prefabricated concrete curbing units as shown on plans and details.

      - Hanover Architectural Products Rock Curb® as marketed by American Hydrotech, Inc. size as noted on plans and details.

      - (Note to Specifier: Per standard product line:  please indicate configuration, layout, size, color, and edge profile of Rock Curb® on plans and details)

  - Checker Block®: as manufactured by Hastings Pavement Co. LLC and as marketed exclusively by American Hydrotech, Inc. meeting the following properties:

Steel reinforced precast concrete

    - Minimum 98 lbs per unit

    - Nominally 24” x 24” x 4” deep

    - Continuous and connected void spaces created by 16 truncated concrete pyramids joined by lower concrete connectors.

      - Stainless steel zip-ties as supplied by American Hydrotech, Inc.

      - Disk Anchors as supplied by American Hydrotech, Inc.

      - (Note to Specifier:  Indicate location of Checker Block units on design drawings and include cross section details to indicate placement of Checker Block units within the Garden Roof assembly.  Contact Hydrotech for specific information on Checker Block layout and available AutoCAD plan and section details.)

- RELATED MATERIALS

  - Metal counterflashing is typically required to provide protection to vulnerable flashing materials from damage due to gardening activities.

PART 3 EXECUTION

- INSPECTION

  - The roofing contractor shall examine all surfaces to receive the roofing assembly to verify it is acceptable and proper for the application of the membrane.

  - The roofing contractor shall not proceed with the installation of the roof membrane assembly until all roof defects have been corrected.

  - Prior to GARDEN ROOF® Assembly installation, a Membrane Integrity Test should be performed prior to the placement of subsequent overburden. If leaks should occur, the water shall be drained completely, and the membrane installation repaired prior to overburden installation. Refer to American Hydrotech's Pre-Installation & Application Guidelines.

- PREPARATION

  - All surfaces must be dry, smooth, free of depressions, voids, protrusions, clean and free of unapproved curing compounds, form release agents and other surface contaminant.

(Note to Specifier: Edit to project requirements)

  - Cast in-place concrete/Composite deck:

Poured in place concrete must be monolithic, smooth, free of voids, spalled areas, laitance, honeycombs, and sharp protrusions.

Refer to SECTION 1.02 of this specification or DIVISION 3.

  - Precast concrete decks

    - Precast units shall be mechanically secured to minimize differential movement and all joints between units shall be grouted or caulked.

    - Joints shall be pre-detailed with membrane and the appropriate manufacturer required reinforcing prior to full membrane application.

  - Plywood decks

Minimum thickness of 1/2" (12.7 mm) shall be required with adequate structural support.

Tongue and groove joint edges shall be required.

Adequate number and type of fasteners shall be used to comply with applicable codes and maintain structural integrity.

  - Metal Deck with roof substrate board

Roof substrate board shall be fire rated type "X" board attached to minimum 22 gauge steel decking with adequate structural support.

      - Minimum 5/8-inch-thick substrate board shall be used directly to metal deck.

      - Minimum 1/2-inch-thick substrate board shall be used over insulation.

Flat fill and tapered roof insulation shall be installed in accordance with layout indicated on shop drawings and insulation board manufacturer’s minimum requirements.

Adequate number and type of fasteners and plates shall be used to comply with roof substrate and insulation board manufacturer’s minimum requirements and applicable codes and to maintain structural integrity.

      - Minimum of 10 fasteners and plates per full-size roof board shall be used.

Appropriate roof adhesive shall be used in accordance with adhesive manufacturer requirements to comply with applicable codes and maintain structural integrity.

      - Size and spacing of adhesive beads shall be as required by adhesive manufacturer.

Roof substrate boards shall be installed such that all edge and end joints are supported by metal deck ribs and/or appropriate blocking.

Roof substrate board end joints of adjacent lengths shall be staggered.

Where required, a suitable vapor barrier membrane shall be applied as determined by the project design professional prior to installation of the roof substrate board and/or insulation.

Roof substrate board edges and ends shall be butt loosely in typical installations. Long, uninterrupted runs (greater than 200 feet) of roof substrate board may require slight gapping due to higher temperature gain. Gapping shall not exceed 3/16 inch and all such gaps shall be filled flush with the surface of the roof board with an appropriate sealant. (Note to Specifier: Contact Hydrotech for specifics.)

Roof substrate board shall be cut to size using a sharp utility knife and straightedge. The surface of the roof substrate board shall be scored with the utility knife and the board bent up sharply towards the score cut. A keyhole-type drywall saw shall be used for penetration cutouts and radii. A low rpm circular saw shall be used for 5/8-inch-thick roof board.

  - Re-Roof/Tear-Off Application

Asphalt, coal tar pitch or other existing membrane must be removed.  (Note to Specifier: CONTACT Hydrotech for specifics.)

Deck type. (Note to Specifier: CONTACT Hydrotech for acceptable Deck Types.)

  - Substrate cleaning

Thoroughly sweep the substrate which is to receive the roof membrane.

Substrate must also be blown clean using an air compressor to remove any remaining loose debris.

Final check to determine if concrete has been properly cleaned is to apply a test patch of Monolithic Membrane 6125® to the surface and check its adhesion.

  - Coordinate Garden Roof® material installation with roofing/ waterproofing applicator.

  - Confirm there are no leaks prior to installation of any Garden Roof® material.

- INSTALLATION

  - Surface conditioner application (to concrete substrates only)

Apply the surface conditioner only to concrete using a handheld sprayer evenly at a rate of 300 to 600 SF/gallon (7.4 - 14.7 m2/L) depending on surface texture.  Surface conditioner shall "tan" the surface, not blacken it.

Allow sufficient time for the surface conditioner to thoroughly dry prior to the membrane application.

  - Membrane preparation

- The membrane shall be heated in double jacketed, oil bath or hot air melter with mechanical agitation, specifically designed for the preparation of a rubberized asphalt membrane.

- Heat membrane until membrane can be drawn-free flowing at a temperature range between 350°F (176°C) and 375°F (190°C).

  - Detailing/Flashing

- All detailing and flashing shall be done in accordance with the manufacturer's standard guideline details.

- All detailing and flashing shall be completed before installing the membrane over the field of the substrate.

- Roof substrate board joints shall be pre-detailed with membrane and fabric reinforcing prior to full fabric reinforced membrane application.

- All liquid-applied, resin flashings shall be applied over properly completed membrane flashing details in accordance with the manufacturer’s standard guideline details.

  - Membrane Application

- Apply the rubberized asphalt membrane at a rate to provide a continuous, monolithic coat of 90 mil minimum (approximately 2.3 mm), into which shall be fully embedded a layer of the spunbonded polyester fabric reinforcing sheet, followed by another continuous monolithic coat of membrane at an average thickness of 125 mil (approx. 3.2 mm).  Total membrane thickness shall be 215 mils average (approx. 5.5 mm), 180 mils minimum.

- Overlap fabric reinforcing sheet 1-2 inches (25.4 mm - 50.8 mm) with membrane between sheets.

- Pre-detailing of joints between plywood and roof substrate board decks shall be required for warranties greater than 10 years.

- SEPARATION/PROTECTION COURSE INSTALLATION

  - Separation/Protection course shall be installed as follows: (Note to Specifier: Edit to project requirements)

  - Hydrocap® 90 separation/protection course shall be embedded into the membrane while it is still hot to insure a good bond. Installation of a separation course shall be necessary to carry out the water test.

    - Overlap adjoining sheet edges (dry) a minimum of 2"-3" (50.8 mm – 76.2 mm) to insure complete coverage.

- MEMBRANE INTEGRITY TEST

- The roof area or portions thereof shall be leak tested by means of Electronic Leak Detection (ELD) or by ponding water at a minimum depth of 2" (50.8 mm) for a period of 48 hours to check the integrity of the membrane installation.

    - ELD testing should be performed on all installed membrane assembly. Testing is to be performed in accordance with ASTM D7877 or ASTM D8231. Areas that are ELD tested should be covered immediately.

    - VERIFY that the structure can support the deadload weight of a water test before testing.

    - If leaks should occur the water shall be drained completely, and the membrane installation repaired.

  - In the event of excessive damage to the membrane assembly, Electronic Leak Detection testing shall be required prior to the placement of subsequent overburden.

- GARDEN ROOF® COMPONENTS INSTALLATION

- All Garden Roof components shall be installed per manufacturer’s requirements.

- Root Barrier Protection. (Note to Specifier: Select appropriate Root Barrier product and edit to project requirements.)

- Root Stop [Root Stop HD] shall be laid over the Hydrocap 90, lapping adjacent sheets with a minimum 30-inch overlap and a minimum 1.75” heat weld to continuously seal the lap edge. Root Stop shall be turned up all vertical roofed/flashed surfaces, installing additional material as required, to completely protect waterproofing and flashings.

- Insulation. Where specified, Styrofoam™ brand insulation shall be installed loose laid in accordance with manufacturer's recommendations.

- Drainage/Water Retention Component. (Note to Specifier: Select appropriate Gardendrain product and edit to project requirements.)

- Gardendrain GR10 [Gardendrain GR25] shall be installed with white fabric surface up, over the root barrier protection, water retention mat (if used) or Styrofoam™ insulation (if used). Adjacent rolls shall be butt together with extra fabric along one edge folded over and lapped onto the adjacent roll. Gardendrain shall be cut to fit around penetrations, etc. with a heavy-duty utility knife.

- Filter Fabric (Note to Specifier: Edit to project requirements)

- Strips of Systemfilter shall be installed at horizontal to vertical transitions at edgings, curbs and penetrations to contain LiteTop engineered growing media. Systemfilter shall overlap adjacent strips a minimum of 12 inches (300 mm) and may be secured with tape during the installation process.

- Systemfilter shall be installed to a height above the finished LiteTop growing media elevation and any excess trimmed down to the level of the plant materials.

- Rock Mineral Wool. (Note to Specifier: Edit to project requirements)

- Install rock wool bats in required area to ensure that rock mineral wool is flat and tightly butted together.

- Do not excessively walk over rock mineral wool after installation.

- If multiple layers of rock mineral wool are required, install subsequent layers at 90 degrees to the one below.

- HARDSCAPE/ACCESSORY INSTALLATION

- Stone and/or paver ballast shall be installed at all roof perimeters, building walls, penetrations, and access hatches, and as required for vegetation free zones, proper wind design, fire breaks, and as walkway/maintenance paths.

- Ballast design shall be in accordance with DuPont™ and American Hydrotech, Inc. requirements. (Note to specifier: CONTACT Hydrotech for ballasting recommendations.)

- Checker Block® shall be installed per Hydrotech requirements and as shown on Hydrotech details.

- Checker Block® shall be installed where indicated on plans and as required per ballasting requirements established by American Hydrotech, Inc.

- Disk Anchors shall be installed in Checker Block® and elsewhere in Garden Roof to the pattern required per the ballasting requirements established by American Hydrotech.

- Stainless steel zip ties as supplied by American Hydrotech shall be used to connect Checker Block® units together as required by American Hydrotech, Inc.

- Metal edge restraints, precast curbing and all specified edging materials shall be installed as shown on plans and details.

- Drains shall be fitted with inspection/maintenance chambers and grills, built up to ensure access at growing media level as shown on plans and details.

- GROWING MEDIA INSTALLATION

- LiteTop growing media shall be placed carefully to avoid damage or displacement of other materials such as walls, paving, drainage components, filter fabric, and roofing membrane.

- LiteTop growing media shall be placed to within 1 inch greater than final grade or to a depth of no greater than 8 inches and compacted as described below. For final grades less than 8 inches only one round of compaction shall be performed and remaining growing media loosely placed such that top of growing media exceeds final grade by 1 inch. For final grades greater than 8 inches, place growing media at no greater than 6 inches and repeat procedure until growing media has been compacted within 1 inch of final grade.

- Compaction shall be performed with a 300 – 400 lb. landscape. Mechanical compactors including plate compactors are not recommended.

- Where Checker Block is installed, roller compaction is not possible. Hand compaction shall be employed to properly compact media. Unless deeper media profile is specified, top of media shall be equal to or just higher than top of Checker Block® units.

- After compaction remaining growing media shall be placed at 1 inch greater than final grade and thoroughly watered or jetted over entire area. Low settled areas shall be filled with additional growing media and re-wet to achieve uniform prescribed final grade.

- Erosion Control Mat. (Note to Specifier: Edit to project requirements)

- The erosion control mat shall be installed directly over the growing media and properly anchored into place.

  - The erosion control mat shall be installed directly over the growing media and properly staked into place with disk anchors.

  - Stake fastening pattern is based on local wind speed, building height and roof slope. Contact Hydrotech for specific guidelines.

- VEGETATION INSTALLATION

- Vegetation planting shall be installed in accordance with the Hydrotech Extensive Garden Roof Plant Installation Guidelines.

- Plant materials shall not be installed between the fall frost date and the following spring frost date.  Contact Hydrotech for fall and spring frost dates specific to the Project and plant material type.

- Growing media shall be thoroughly watered and saturated immediately prior to installing new plant material.

- Where Checker Block® is installed, media shall be properly placed before plant material is installed. Where InstaGreen sedum carpet is used, no gaps shall exist between the bottom of the sedum carpet and the media placed in the Checker Block® units.

- All vegetation shall be thoroughly watered and saturated immediately after installation. Media is saturated when water is seen flowing into roof drains from adjacent areas.

- VEGETATION MAINTENANCE

- Contractor/Installer shall maintain plantings in accordance with the Hydrotech Extensive Garden Roof Maintenance Guidelines.  Contact Hydrotech for specific maintenance requirements.

- Maintenance activities shall include, but are not limited to, the following:

- Periodic on-roof monitoring of vegetation

- Watering to maintain proper growing media moisture content (especially during periods of hot and dry weather)

- Weeding to remove unwanted vegetation from planted areas and vegetation free zones.

- Removal of debris

- Reporting and photo-documentation of progress of vegetation during maintenance and warranty period.

- Maintenance shall begin immediately after vegetation installation and shall continue through final acceptance and turn-over of the project to the owner.

- Maintenance activities shall continue throughout the two-year warranty period (from date of vegetation installation) to keep vegetation warranty referenced above under Paragraph 1.10 WARRANTY in effect.

END OF SECTION